fall 2024

Cse-4355/5355 Electromechanical Systems and Sensors

the university of texas at arlington

lab 2 report

Submitted toward the partial completion of the requirements for CSE 4355/5355

Under the guidance of

Prof. Jason Losh

By:

Dario Ugalde, 1001268068

Joshua Martinez, 1001374818

## Introduction

The typically electric motor is powered by DC current. These devices require internal brushes, which normally consist of carbon or graphite, for operation. They deliver power to the motor causing it to spin through magnetization and EMF. The permanent magnets inside pull the energized portions of the motor based on the brushes. Over time, brushes wear away during usage. Brushed DC motors thus have a limited lifecycle based on the longevity of the internal brushes. Another sort of motor this lab is based upon are stepper motors and are considered brushless motors. They swap the usage of brushes for electronic control. Despite the loss of ease of use and build, these types of motor gain the advantages of a longer lifespan and greater energy efficiency. Based on the digital inputs they receive, certain poles become energized. Through the necessary pulsing of these inputs, the shaft inside shall rotate accordingly. Based on the phases, or steps, around the coil, determines the discrete angular movement from one phase to another. Hence the name, stepper motor. This lab explores the usage of stepper motors and knowledge necessary to operate it.

## Steps 1-5: Initial Understanding and Setup

The initial steps of this lab explore the usage and connectivity of the stepper motor. As described on the datasheet, the motor’s voltage rating is 10.4V, or the safest maximum voltage it can withstand. This limit will be used during experimentation.  Within the motor, there are two sides requiring voltage input.  The motor has two coils, known as Coil A and Coil B which, for this lab, power/magnetize the shaft and the outer ring. When one of the coils receives an input, it moves the shaft into the pulled position. The strength of the shafts position correlates to the voltage applied.

The next few steps require measuring the holding torque of the motor. A metal bar is attached to the shaft in its center. Using a 225g weight, we can measure the holding torque of the motor. At 10.4V, it held in place up to .115m from the center. At 9.4V, it held up to .105m, and .095m at 8.4V. Thus, the holding torque seems to be proportionally linear to the voltage applied to the coil. The next step requires no weight, but understanding the rotation, or steps, of the motor depending on the voltage connection pattern. With two coils, each with positive and negative ends, there are a total of 4 inputs. The described pattern is coil ‘A’ positive, ‘B’ positive, ‘A’ negative, and ‘B’ negative. Doing this order turned the motor clockwise.  When performing this routine backwards, the motor rotated counterclockwise. Through the repetitive usage of these patterns, we can set the rotation distance of the motor.

## Steps 6 – 9: Driving the Stepper Motor using half H-Bridges

The next set of steps introduces four half H-bridges to allow operation of the motor with the TM4C microcontroller. On the motor side of the bridges, they receive the 10.4 volts and ground from the power supply. The four outputs from this side connect one each to the mentioned 4 inputs of the coils. On the logic side of the bridges, it receives the voltage and ground from the red board. There are four motor control connections from this side for the microcontroller to control. These receive signals to power the corresponding coil and the polarity. Lastly, each bridge has an enable input that they use to power on. Two of the bridges controlling the same coil will be driven from one input each. The expected circuit results in the TM4C using 6 output pins to turn the motor on via code: 2 enable pins, and 1 pin for each of the 4 coil polarity connections. The enable pins require PWM control, but for these steps, being set as GPIO signals will suffice.

The next step necessitates for code running on the TM4C to control the outputs. The desired outcome here is to mimic a rocking motion with the metal bar. This is done through the patterns mentioned earlier. Through the implementation of a while loop, each step of a pattern could be replicated through the output pins followed by a second delay between them. When done properly, the motor will rotate the bar in one direction. Adding the other pattern in series, or the inverse, will rotate in the opposing direction. These two together will mimic the rocking motion to ensure proper connection and alignment from the microcontroller to the motor.

## Steps 10-11: Sensor inclusion for a Zero Degree base

Before the final steps, the integration of an optical switch will be needed. It contains an IR LED and an NPN phototransistor. The LED’s function is to turn the NPN on when unencumbered. Therefore, when the motor/bar moves a certain distance in the direction of the switch, they will block the LED light with a brass tab aligned with the bar. The purpose of this switch is to aid in leveling the metal bar to a zero degree stop. For the optical switch to work it needs the LED cathode and NPN emitter to be grounded. The anode and collector are powered from the board’s 3.3V with 220 and 47K ohms respectively. The board will read the power off the switch via a GPIO input on the collector. Using the rocking motion code as a basis, a segment will be added which allows the switch to notify the program, a signal change from the phototransistor, the motor/bar have moved to the required location to block the LED. Afterwards for about 22 steps, the bar should reach the desired zero-degree outcome. Once completed, despite the bars location during start up, the motor will rotate the bar clockwise until it hits the end/switch and indicates the microcontroller to move the bar in the opposite direction until its leveled.

## Steps 12-14: Micro-stepping the Motor via PWM Function

Finally, the last few steps for this lab require two new main functions. For the first, we must build off the zero-degree base program to rotate the bar to the user’s desired degree. With the aid of UART, said users will send either a positive or negative number for the running program to receive and run. Based on the polarity, the motor must rotate according to the received input value. The motor must move to represent each valid degree within range, about 75 degrees total. Rather than resetting the bar to zero after each command, code will be needed to remember the location, or angle, of the motor as the starting placement for the next valid input.

The second function needed is the modification of the step sizes and amount with the usage of PWM on the enable outputs. To implement these micro steps, understanding the full steps is required. The current four steps have one coil powered on at a single time, and only one polar side as well. A coil has three phases: positive, zero, and negative. Using the clockwise pattern as well as Coil A and B as X and Y coordinates, respectively, they list out: (1,0), (0,1), (-1,0), and (0,-1). Thus, the pattern can be seen as 4 maximums of a circle. At either of these points are two coils on at a time. To summarize, at each of the four steps, only one coil has an input while the other is turned off. To add micro steps, the motor must transition from one maximum to the other. This is accomplished through powering off one coil and energizing the other with the correct polarity. The transitions the coils receive can be represented by cosine and sine. Depending on the changing of degrees, one function will climb to one and the other falls to zero. With the PWM functionality, the enable inputs can be multiplied by cosine and sine, when done correctly, to achieve these transitions. The smaller the degree change and steps between the maximums will determine how smooth the motor turns and completing the requirements for the last function.

## Circuit and Final Setup

Figure 14: Complete Setup Used in Lab